Table of Contents

# Mono FM Receiver — Complete FPGA SoC Implementation

Marco Aiello
Principal FPGA Design and Verification Engineer
Huntingdon, Cambridgeshire, UK

Target device: Xilinx Zynq-7010 (xc7z010clg400-1) · Digilent Zybo Z7-10
Toolchain: Vivado / Vitis HLS / PetaLinux 2022.2 · MATLAB R2022b
Status: Hardware verified · Audio confirmed on target

## Executive Summary

This project implements a complete, hardware-verified software-defined FM radio receiver on a Xilinx Zynq-7010 SoC. Starting from a real RTL-SDR I/Q baseband recording of a live FM broadcast, the design demodulates the signal entirely in FPGA programmable logic and delivers decoded 48 kHz PCM audio to an SD card WAV file.

The implementation is distinguished by engineering rigour at every stage. A MATLAB floating-point prototype was converted to a Simulink fixed-point model — verified to greater than 40 dB PSNR against the floating-point reference on real broadcast content — before a single line of RTL was written. Fixed-point word lengths were determined analytically and empirically using MATLAB Fixed-Point Designer, then overridden in four specific cases where the auto-proposer’s empirical range collection gave provably wrong answers. The RTL implementation follows a four-level verification hierarchy using HLS C-simulation, RTL co-simulation, full chain behavioural simulation with MATLAB PSNR gating, and a PS7 VIP end-system testbench that reproduces hardware conditions in simulation. A hardware bring-up fault (wrong WAV content on target) was isolated to a cache coherency bug using the simulation testbench before a single line of code was changed.

The project spans the full embedded FPGA design stack: fixed-point DSP algorithm development, Vitis HLS with custom pragmas and fixed-point type discipline, hand-written VHDL for timing-critical blocks, Vivado IP Integrator with a fully scripted build flow, AXI DMA architecture, bare-metal embedded C with cache maintenance, and a PetaLinux Linux port. Everything is reproducible from a clean checkout using a single build script.

## Technical Scope at a Glance

| Aspect | Detail |
| --- | --- |
| Device | Xilinx Zynq-7010, xc7z010clg400-1 |
| Clock | 100 MHz (PL), FCLK_CLK0 from PS7 |
| Signal chain | 9 processing stages, 100% custom |
| DSP48 utilisation | 28 / 80 (35%) — tight fit by design |
| Timing closure | WNS +0.432 ns @ 100 MHz, zero failing endpoints |
| Input | 250 kHz I/Q baseband, RTL-SDR capture, sfix16_En15 |
| Output | 48 kHz mono PCM WAV via polyphase FIR resampler |
| HLS blocks | 4 (AA LPF ×2, FM disc, audio ppdma, IQ ppdma) |
| VHDL blocks | 5 (freq_corr, NCO wrapper, de_emph, iq_splitter, tlast_gen) |
| Xilinx IPs | DDS Compiler v6.0, FIR Compiler v7.2 ×2, AXI GPIO |
| Verification levels | 6 (csim, cosim, AXI-S VIP, chain sim+PSNR, PS7 liveness, PS7 audio check) |
| Software | Bare-metal C (FATFS, Xil BSP) + PetaLinux Linux port |
| Build | Fully headless: MATLAB → HLS → Vivado → Vitis → PetaLinux |

## 1. The Input Signal and Why the 10 kHz Offset Matters

The input recording rds.wav was captured using an RTL-SDR Blog V3 dongle and SDRuno, tuned to receive WAY-FM at 88.100 MHz. The SDRuno local oscillator was deliberately set 10 kHz above the station at 88.110 MHz. This is standard practice with RTL-SDR hardware: the direct-conversion front end produces a DC spike at exactly 0 Hz in the baseband caused by LO leakage and self-mixing. Tuning the LO 10 kHz away from the station moves the DC spike to −10 kHz in the baseband, well clear of the audio content. The station carrier therefore sits at −10 kHz in the recorded baseband rather than at DC.

The RDS decoder in SDRuno confirmed the recording: PI code 22301, programme service “WayFM”, PTY “Top 40”, RadioText “WAY-FM Uplifting. Upbeat.” The signal carries the full MPX multiplex — pilot tone at 19 kHz, stereo difference subcarrier at 38 kHz DSB, RDS data at 57 kHz BPSK — though this implementation decodes only the mono L+R sum.

The 10 kHz offset drives the first design decision: the NCO in the FPGA is configured to rotate the baseband by exactly +10 kHz, restoring the station carrier to DC before any other processing. The DDS Compiler phase increment for this is:

PINC = round(f_offset / f_s × 2^phase_width)
     = round(10000 / 250000 × 2^24)
     = 671089

This value is hardcoded in the DDS Compiler IP configuration. The design supports dynamic retuning via the DDS Compiler’s S_AXIS_CONFIG port, which would allow software to adjust the NCO frequency without resynthesis — the foundation of an automatic frequency control (AFC) loop as a future extension.

## 2. System Architecture

### 2.1 Signal Chain

The complete demodulation pipeline runs in the Zynq-7010 PL at 100 MHz. All stages are synchronous to FCLK_CLK0 generated by the PS7 block.

rds.wav (250 kHz I/Q, sfix16_En15)
    │
    ▼
┌─────────────────────────────────────────────────────────────────┐
│  iq_ppdma_0                                                     │
│  Vitis HLS · DDR ping-pong → AXI-Stream                        │
│  I[15:0] = packed[31:16]    Q[15:0] = packed[15:0]            │
└─────┬──────────────────────────────┬──────────────────────────┘
      │                              │
      ▼                              │
┌─────────────┐   ┌────────────────┐ │
│ nco_wrapper │   │  iq_splitter   │◄┘
│ DDS +10 kHz │   │ 32b→two 16b   │
│ sfix16_En15 │   │ AXI-S streams │
└──────┬──────┘   └────────┬───────┘
  cos  │ sin            Ic │ Qc
       └──────┬────────────┘
              ▼
    ┌──────────────────┐
    │   freq_corr      │  VHDL · 2-cycle pipeline
    │  Ic = I·cos−Q·sin│  sfix16_En15 → sfix18_En17
    │  Qc = I·sin+Q·cos│
    └──────┬───────────┘
           │ Ic             │ Qc
    ┌──────▼──────┐  ┌──────▼──────┐
    │  aa_lpf_I   │  │  aa_lpf_Q   │  Vitis HLS · 129-tap FIR
    │ Kaiser β=8  │  │ Kaiser β=8  │  cutoff 100 kHz
    └──────┬──────┘  └──────┬──────┘  sfix18_En17
           └────────┬────────┘
                    ▼
          ┌──────────────────┐
          │    fm_disc       │  Vitis HLS · 16-iter CORDIC
          │ atan2(Qdiff,Idiff│  × 39788.7358 Hz/rad
          └────────┬─────────┘  sfix32_En14 @ 250 kHz
                   │
          ┌────────▼─────────┐
          │  fir_dec  R=5    │  FIR Compiler · polyphase
          │  250→50 kHz      │  sfix32_En14 @ 50 kHz
          └────────┬─────────┘
                   │
          ┌────────▼─────────┐
          │   audio_lpf      │  FIR Compiler · 127-tap
          │   15 kHz passband│  sfix32_En14 @ 50 kHz
          └────────┬─────────┘
                   │
          ┌────────▼─────────┐
          │   de_emph        │  VHDL · 2-stage IIR
          │  τ=75µs, b0=0.234│  sfix32_En13 @ 50 kHz
          │  a1=0.766        │
          └────────┬─────────┘
                   │
          ┌────────▼─────────┐
          │  tlast_gen       │  VHDL · DMA framing
          └────────┬─────────┘
                   │
          ┌────────▼─────────┐
          │ audio_ppdma_0    │  Vitis HLS · AXI-S→DDR
          │ ping-pong + mirror│  50 samples/buffer (1 ms)
          └────────┬─────────┘
                   │ HP0 (non-coherent)
                   ▼
              DDR3L 0x3E400000
                   │
                   ▼ (CPU polls GPIO active_buf,
          ARM Cortex-A9      invalidates cache, reads via memcpy)
          PCM16 conversion → f_write → SD card
          resample 50k→48k → audio_out.wav

### 2.2 Fixed-Point Format Map

Every inter-stage boundary carries a precisely defined fixed-point format. This table was the governing specification for both the Simulink model and the RTL implementation:

| Stage | Format | W | I | F | Range | Rate |
| --- | --- | --- | --- | --- | --- | --- |
| ADC / input | sfix16_En15 | 16 | 1 | 15 | (−1, +1) | 250 kHz |
| NCO output | sfix16_En15 | 16 | 1 | 15 | (−1, +1) | 250 kHz |
| FreqCorr output | sfix18_En17 | 18 | 1 | 17 | (−1, +1) | 250 kHz |
| AA LPF output | sfix18_En17 | 18 | 1 | 17 | (−1, +1) | 250 kHz |
| FM Disc output | sfix32_En14 | 32 | 18 | 14 | ±131072 Hz | 250 kHz |
| FIR Dec output | sfix32_En14 | 32 | 18 | 14 | ±131072 Hz | 50 kHz |
| Audio LPF output | sfix32_En14 | 32 | 18 | 14 | ±131072 Hz | 50 kHz |
| De-emph output | sfix32_En13 | 32 | 19 | 13 | ±262144 | 50 kHz |
| PCM16 output | int16 | 16 | 16 | 0 | ±32767 | 48 kHz |

The FM discriminator output requires 18 integer bits because the maximum instantaneous deviation of ±75 kHz scaled by fs/(2π) = 39788.7 gives ±75000 × (250000/2π) / 250000 ≈ ±75 kHz, which needs ceil(log₂(131072)) + 1 = 18 bits including the sign bit. Choosing sfix18_En15 (as Fixed-Point Designer initially proposed) would overflow silently on any signal deviating more than approximately ±3 kHz — a catastrophic precision loss caught by the PSNR verification.

### 2.3 SoC Block Design Hierarchy

The block design is split into two levels, each packaged as its own Vivado IP:

Inner BD — fm_demod: contains the entire signal processing chain from IQ input to audio output. Packaged as fm_demod_axis_with_sidechannels:1.0. Exposes three AXI-Stream interfaces: s_axis_i_0 (16-bit, 250 kHz), s_axis_q_0 (16-bit, 250 kHz), m_axis_data_0 (32-bit, 50 kHz).

Outer BD — sdr_fm_receiver: the full SoC. Instantiates fm_demod_0, the two HLS DMA cores, PS7, AXI interconnects, GPIO, reset controller, and all xlconstant tie-off blocks.

This naming was deliberate. An earlier version used Vivado’s default name design_1 for both block designs, causing XSim to instantiate the inner wrapper (design_1_wrapper) instead of the outer SoC wrapper during elaboration — both modules shared the same name, and XSim used the first one compiled with no error. The failure produced complete silence on all AXI buses, indistinguishable from an undriven reset until the elaboration log’s “already exists in library work” warning was identified.

### 2.4 AXI Bus Topology

Three distinct AXI bus types, each with a specific role:

M_AXI_GP0 (32-bit general purpose): the ARM CPU’s outbound AXI master. Used for all control-plane transactions: reading GPIO registers, writing to HLS AXI-Lite control registers. Carries negligible bandwidth — a 32-bit register read every few milliseconds.

S_AXI_HP0 (64-bit high performance): the PL’s high-throughput inbound path to the DDR controller. Both iq_ppdma_0 and audio_ppdma_0 use this port for all data movement. Bypasses the ARM L1/L2 data cache — the source of the hardware bring-up bug described in Section 7.

AXI4-Stream: all inter-IP connections within the PL. No address, no CPU involvement during steady-state operation. Every connection between signal-processing stages uses AXI4-Stream with tvalid/tready handshaking.

### 2.5 Address Map

| Peripheral | Base Address | Range |
| --- | --- | --- |
| AXI GPIO | 0x4000_0000 | 64 KB |
| iq_ppdma_0 AXI-Lite | 0x4001_0000 | 64 KB |
| audio_ppdma_0 AXI-Lite | 0x4002_0000 | 64 KB |
| IQ PING buffer | 0x3E00_0000 | 1 MB |
| IQ PONG buffer | 0x3E10_0000 | 1 MB |
| Audio PING buffer | 0x3E20_0000 | 1 MB |
| Audio PONG buffer | 0x3E30_0000 | 1 MB |
| Audio DEST (mirror) | 0x3E40_0000 | 1 MB |

The five DDR regions from 0x3E000000 to 0x3E4FFFFF are declared as no-map reserved memory in the PetaLinux device tree (reserved-memory.dtsi), preventing the Linux kernel’s buddy allocator from ever allocating those pages to unrelated processes.

## 3. Design Methodology: Model-Based Design

### 3.1 The Four-Stage Pipeline

This project follows MathWorks’ Model-Based Design pipeline. Each stage answers a specific question before the next begins, and the transition between stages is gated by a quantitative verification criterion.

Stage 1 — MATLAB floating-point algorithm: answers “does the signal processing algorithm work?” run_and_extract.m loads rds.wav, normalises to fixdt(1,16,15), implements the full demodulation chain in double-precision MATLAB, and saves intermediate node vectors to ic_qc_gold.mat. This stage produces the definitive reference output — the floating-point ground truth that all subsequent stages are measured against.

Stage 2 — Simulink fixed-point model: answers “does the algorithm work as a connected system with the proposed word lengths?” The model fm_demod_slx.slx implements all six processing stages as Simulink fixed-point blocks with types set by Fixed-Point Designer. Key model parameters confirmed from the .slx XML: solver FixedStepDiscrete at 4 µs step (250 kHz base rate); FM disc subsystem contains 2 UnitDelay blocks, 4 Product blocks, 2 Sum blocks, a Trigonometry(atan2) block, and a Gain of exactly 39788.7358; de-emphasis DiscreteFilter with b0 = 0.234071661635351 and a1 = 0.765928338364649. The ToWorkspace block audio_50k is configured with FixptAsFi=on, preserving the sfix32_En13 type information in the workspace variable.

Stage 3 — Fixed-Point Designer word-length analysis: answers “what is the minimum word length at each node that meets the quality specification?” Fixed-Point Designer runs range collection on the real rds.wav stimulus and proposes minimum word lengths to meet a 40 dB PSNR target. Four proposals were manually overridden — see Section 4.

Stage 4 — HLS and VHDL implementation: answers “how is this built in hardware?” With all word lengths verified in Simulink, the RTL implementation begins. The parallel workstream pattern (see Section 5) allows HLS blocks to be developed and verified concurrently while the Simulink model is being refined.

### 3.2 The Simulink Model Internals

The Simulink model’s block diagram, recovered from the .slx XML, is reproduced here:

FromWorkspace(ts_I)  ─┐
FromWorkspace(ts_Q)  ─┤
FromWorkspace(ts_cos)─┼──► [FreqCorr SubSystem] ──► AA_I (DiscreteFir 129t)
FromWorkspace(ts_sin)─┘                          └──► AA_Q (DiscreteFir 129t)
                                                       │
                                                       ▼
                                              [FM_Disc SubSystem]
                                              Id=UnitDelay(Ic)
                                              Qd=UnitDelay(Qc)
                                              Idiff = Ic·Id + Qc·Qd
                                              Qdiff = Qc·Id − Ic·Qd
                                              atan2(Qdiff,Idiff)
                                              × Gain(39788.7358)
                                                       │
                                              Buffer(N=5) ◄── required for
                                                       │       frame-based path
                                              FIR Decimation R=5
                                                       │
                                              DataTypeConversion (Floor)
                                                       │
                                              AudioLPF (DiscreteFir 127t)
                                                       │
                                              DeEmph (DiscreteFilter)
                                              b0=0.234072, a1=0.765928
                                                       │
                                              ToWorkspace(audio_50k, FixptAsFi=on)

The Buffer N=5 block is a design detail that is not apparent from the scripts alone — the DSP Toolbox FIR Decimation block in frame-based mode requires a frame of exactly N=D=5 samples at its input. The Buffer block converts the scalar stream from FM_Disc into 5-element frames at the decimator input.

The DataTypeConversion block between the FIR Decimation and AudioLPF performs the explicit format conversion from the decimator’s internal accumulator output back to sfix32_En14. Its rounding mode is Floor (truncation), matching the Xilinx FIR Compiler’s configured rounding mode.

### 3.3 The Pre-Sim / Post-Sim Handoff

run_and_extract.m follows a strict three-phase pattern:

Pre-sim: loads rds.wav, quantises to fixdt(1,16,15), computes the NCO cosine/sine timeseries analytically (cos(2π × 10000 × t), sin(2π × 10000 × t)), packs all four into timeseries objects. The NCO is computed in MATLAB rather than modelled as a Simulink DDS block — this is a pragmatic choice: the NCO timeseries is a trivial MATLAB one-liner, and implementing it in Simulink would add complexity with no benefit to the fixed-point analysis of the downstream stages.

sim(mdl): Simulink runs the complete model. All fixed-point arithmetic, all block-level saturation and rounding, all pipeline behaviour executes here.

Post-sim: extracts ToWorkspace variables, saves ic_gold/qc_gold to ic_qc_gold.mat for the RTL workstream, applies the 50k→48k resample using MATLAB’s resample(audio, 24, 25) (the FPGA does not implement this — it outputs at 50 kHz and software handles the rate conversion), and plays the audio via sound() for subjective confirmation.

## 4. Fixed-Point Design: Word-Length Selection and Overrides

### 4.1 Why FPGA Fixed-Point Is Different From DSP Fixed-Point

On a TI C6000 DSP or an ARM Cortex-A9, the word length is fixed by the hardware architecture. Every intermediate computation occupies a 16-bit, 32-bit, or 64-bit register — the extra bits are there whether or not they carry useful signal. On an FPGA, every wire has exactly as many bits as the designer specifies. A 40-bit accumulator occupies exactly 40 bits of flip-flop resources; a 32-bit bus occupies 32 wires. This means that choosing the minimum adequate word length at each node directly translates to reduced DSP48 usage, reduced routing congestion, and reduced power.

The Z7-10 has 80 DSP48E1 slices. The FM receiver uses 28. A naive implementation using sfix32 arithmetic everywhere for “safety” would have required approximately 60–70 DSP48 slices for the AA LPF alone, exceeding the device’s budget before the FM discriminator was added. Fixed-Point Designer’s word-length minimisation is not a theoretical exercise — it is what makes the design fit on the target device.

### 4.2 The Four Manual Overrides

Fixed-Point Designer’s auto-proposer was wrong in four specific cases. The table below documents each override and the reasoning:

Override 1 — Input format: sfix16_En4 → sfix16_En15

The auto-proposer observed the range of the floating-point simulation data (values in (−1, +1)) and estimated the format correctly. Then the original conversion script unconditionally replaced the proposal with sfix16_En4 — a 16-bit format with 4 fraction bits and 12 integer bits, representing values up to ±2048. This quantises the ADC input to 32 distinct levels (equivalent to a 5-bit ADC) before any processing. The audio was catastrophically distorted. The correct format is sfix16_En15, matching the ADC’s actual normalised output range of (−1, +1) exactly.

Override 2 — Discriminator output: sfix18_En15 → sfix32_En14

The auto-proposer proposed sfix18_En15 for the CORDIC atan2 output, matching the Simulink CORDIC block’s output precision. It did not account for the immediately following Gain(39788.7358) block. The gain block’s output must represent instantaneous frequency in Hz — maximum ±75 kHz per broadcast standard, with margin for off-tuned carriers. This requires ceil(log₂(130000)) + 1 = 18 integer bits, for a total of 32 bits at sfix32_En14. With sfix18_En15, the gain block overflows silently for any signal with more than ~3 kHz of instantaneous frequency deviation — which is the case for virtually every real FM broadcast.

Override 3 — De-emphasis accumulator: narrow → sfix40_En13

The auto-proposer proposed a narrow accumulator based on the steady-state signal range during simulation. An IIR filter with a feedback coefficient close to 1 (a1 = 0.766) has a long memory — the worst-case transient (a step change from silence to full-scale audio) produces a much larger accumulator value than the steady-state. Additionally, a narrow accumulator in an IIR with a1 close to 1 produces limit cycles: persistent low-level oscillations in the output when the input is zero, caused by rounding error circulating in the feedback loop. The 40-bit accumulator fits in two DSP48 slices and eliminates this risk analytically.

Override 4 — Product block default: Inherited → Explicit sfix32

Simulink’s Product block defaults to “Inherit via internal rule” for the output type. For two sfix18_En17 inputs, this internal rule resolves to something like sfix18 — truncating the 36-bit full-precision product to 18 bits and discarding exactly half the precision of the multiplication. This was set explicitly to sfix32 throughout the frequency corrector and FM discriminator subsystems, where maintaining intermediate precision before the final accumulation step matters for audio quality.

## 5. RTL and HLS Implementation: Every Block in Depth

### 5.1 NCO Wrapper (nco_wrapper.vhd)

The NCO uses the Xilinx DDS Compiler v6.0 IP rather than a hand-written ROM-based NCO, for three reasons: the DDS Compiler applies a Taylor series correction to suppress spurious harmonics to a specified SFDR level; its PINC-to-frequency relationship is pre-verified and documented; and its S_AXIS_CONFIG port supports runtime frequency updates without resynthesis, enabling a future AFC loop.

The DDS Compiler’s tdata packing per PG141 is:

tdata[15:0]  = cosine   (sfix16_En15)
tdata[31:16] = sine     (sfix16_En15)

Cosine occupies the low 16 bits and sine the high 16 bits — the opposite of the intuitive arrangement. A bring-up bug caused by reading this incorrectly (swapping the slice assignments) rotated the frequency-corrected baseband by 90°, producing zero discriminator output. The wrapper splits this packed word into two separate 16-bit AXI-Stream outputs (m_axis_cos, m_axis_sin), each with its own tvalid.

### 5.2 Frequency Corrector (freq_corr.vhd)

The frequency corrector performs the standard complex baseband mixing operation:

Ic[n] = I[n] · cos(θ[n]) − Q[n] · sin(θ[n])
Qc[n] = I[n] · sin(θ[n]) + Q[n] · cos(θ[n])

This is a rotation of the complex phasor (I + jQ) by angle θ[n], exactly cancelling the −10 kHz rotation accumulated in rds.wav.

Interface design: The block exposes four separate AXI-Stream slave ports (one per scalar input: I, Q, cos, sin) rather than a single packed bus. This matches the Simulink FreqCorr subsystem’s four independent inputs. The all-valid handshake is the critical design decision:

in_valid <= s_axis_i_tvalid AND s_axis_q_tvalid AND
            s_axis_cos_tvalid AND s_axis_sin_tvalid;

Only when all four inputs simultaneously present valid data does the block accept any of them. This prevents I/Q skew — if any one of the four input streams stalls due to backpressure, no inputs advance. A 1-sample skew between I and Q would rotate the discriminator’s input phasor by an amount proportional to the signal frequency, introducing distortion that is impossible to remove downstream.

Pipeline: 2 clock cycles. Cycle 1 latches all four inputs into registers. Cycle 2 computes the four 16×16 multiplications, two 33-bit sums, and the right-shift-and-saturate to 18 bits. The word grows from sfix16_En15 to sfix18_En17 to accommodate the carry from the sum of two sfix17_En15 products.

### 5.3 Anti-Alias LPF (aa_lpf.cpp, aa_lpf.h)

The AA LPF is a 129-tap symmetric FIR lowpass filter applied independently to the I and Q channels. Its purpose is to limit the I/Q bandwidth before the FM discriminator.

Why lowpass, and why 100 kHz cutoff: The RTL-SDR captures ±125 kHz of bandwidth. After frequency correction, the station sits at DC, but adjacent channels, pilot tones, and wideband noise occupy the full ±125 kHz window. The FM discriminator computes the phase derivative of the entire complex spectrum — feeding it 250 kHz of bandwidth means it differentiates the phase of adjacent channel interference alongside the desired signal. The AA LPF cuts to ±100 kHz before the discriminator. The 100 kHz cutoff (rather than the theoretical ±75 kHz FM deviation maximum) provides a 25 kHz transition band that does not attenuate the outermost FM sidebands during loud passages.

Filter design: fir1(128, 100e3/125e3, 'low', kaiser(129, 8)) — Kaiser window with β=8, providing approximately 80 dB of stopband attenuation, substantially more than the 40 dB PSNR target requires.

Fixed-point types from aa_lpf.h:

typedef ap_fixed<18, 1, AP_TRN, AP_SAT> data_t;   // sfix18_En17  I/O
typedef ap_fixed<32, 2>                  coef_t;   // sfix32_En30  coefficients
typedef ap_fixed<40, 23, AP_TRN, AP_SAT> acc_t;   // sfix40_En17  accumulator
#define NTAPS 129

The 40-bit accumulator is wider than the theoretical 50-bit full-precision product (18+32=50) but sufficient in practice: a lowpass FIR with unity DC gain has coefficient magnitudes summing to approximately 1.0, so the accumulator never reaches its theoretical maximum on any real signal. Fixed-Point Designer confirmed this empirically from the rds.wav stimulus.

The critical pipeline pragma decision: #pragma HLS PIPELINE II=1 is placed on the loop, not the function. The distinction matters enormously:

MAC_Loop:
for (int i = 0; i < NTAPS; i++) {
    #pragma HLS PIPELINE II=1    // <-- on the loop
    acc += shift_reg[i] * c[i];
}

Placing PIPELINE on the loop schedules 129 sequential MAC operations through 129 cycles, reusing a small set of DSP48 instances (approximately 9 per channel). Placing it on the enclosing function would fully unroll the loop, instantiating up to 129 parallel multipliers — requiring approximately 118 DSP48 slices for the two channels combined, nearly 1.5× the entire device budget.

The by-reference pragma requirement: x must be declared as data_t &x (reference), not data_t x (value):

// WRONG: generates ap_none port with no tvalid/tready
void aa_lpf(data_t x, data_t &y);

// CORRECT: generates proper AXI-Stream interface
void aa_lpf(data_t &x, data_t &y);

HLS’s mode=axis interface can only generate AXI-Stream streaming hardware for reference parameters — a pass-by-value scalar silently falls back to ap_none (plain wire, no handshake). This was a real bug that compiled and synthesised without error, producing a port with no tvalid or tready.

### 5.4 FM Discriminator (fm_disc.cpp, fm_disc.h)

The FM discriminator implements the standard differential-phase detection algorithm with a fixed-point CORDIC atan2 function. The algorithm computes the phase difference between consecutive samples as a proxy for instantaneous frequency.

Algorithm:

// Cross-products: conjugate multiply of current and delayed sample
prod_t x_diff = ic * id + qc * qd;   // Idiff: real part
prod_t y_diff = qc * id - ic * qd;   // Qdiff: imaginary part

// CORDIC atan2 in vectoring mode
phase_t phase = cordic_atan2(cordic_t(x_diff), cordic_t(y_diff));

// Scale radians/sample to Hz
const out_t r2hz = 39788.7358;   // fs / (2π) = 250000 / (2π)
disc_out = out_t(phase * r2hz);

// Update one-sample delay registers
id = ic;   qd = qc;

Fixed-point type chain:

typedef ap_fixed<18,  1, AP_TRN, AP_SAT> data_t;    // input: sfix18_En17
typedef ap_fixed<18,  4, AP_TRN, AP_SAT> prod_t;    // cross-products: sfix18_En14
typedef ap_fixed<32,  8, AP_TRN, AP_SAT> cordic_t;  // CORDIC internal
typedef ap_fixed<18,  3, AP_TRN, AP_SAT> phase_t;   // CORDIC output: sfix18_En15
typedef ap_fixed<32, 18, AP_TRN, AP_SAT> out_t;     // output: sfix32_En14
#define CORDIC_ITER 16

The CORDIC implementation: 16 iterations, fully unrolled with #pragma HLS UNROLL. Each iteration:

cordic_t xs = x * SHIFT[i];   // x × 2^-i (synthesises as bit-shift)
cordic_t ys = y * SHIFT[i];   // y × 2^-i
if (y >= 0) { x_new=x+ys; y_new=y-xs; z=z+ATAN_LUT[i]; }
else        { x_new=x-ys; y_new=y+xs; z=z-ATAN_LUT[i]; }

The SHIFT[i] constants (1.0, 0.5, 0.25, ...) synthesise as bit-shifts, not multipliers. The ATAN_LUT is a lookup table of atan(2^-i) values precomputed at synthesis time as phase_t constants. The result: 16 iterations using only adders and constant-shift wiring — no DSP48 slices consumed by the CORDIC itself.

The ATAN_LUT from the actual source:

static const phase_t ATAN_LUT[CORDIC_ITER] = {
    phase_t(0.7853981634),   // atan(2^0)  = π/4
    phase_t(0.4636476090),   // atan(2^-1)
    phase_t(0.2449786631),   // atan(2^-2)
    phase_t(0.1243549945),   // atan(2^-3)
    phase_t(0.0624188100),   // atan(2^-4)
    phase_t(0.0312398334),   // atan(2^-5)
    ...
    phase_t(0.0000305176),   // atan(2^-15)
};

Initial quadrant normalisation extends the range from (-π/2, +π/2) to the full (-π, +π) without adding iterations:

if (x < 0) {
    x = -x_in;  y = -y_in;
    z = (y >= 0) ? phase_t(3.14159265359) : -phase_t(3.14159265359);
}

CORDIC gain: the iterative rotation applies a gain of ≈1.6468× to the input vector magnitude. This gain is not compensated — it is matched to the Simulink CORDIC block’s behaviour. The r2Hz constant (39788.7358) was calibrated against the full gain path including this factor.

### 5.5 De-emphasis IIR (de_emph.vhd)

The de-emphasis filter implements the ITU-R broadcast standard’s RC de-emphasis characteristic with a 75 µs time constant (Americas). The difference equation:

y[n] = b0 · x[n] + a1 · y[n-1]

with coefficients derived from exp(-T/τ):

τ = 75 × 10⁻⁶ s,  fs = 50000 Hz,  T = 1/fs = 20 µs
a1 = exp(-T/τ) = exp(-0.2667) = 0.765928338364649
b0 = 1 - a1    = 0.234071661635351

Quantised to sfix32_En14:

constant B0 : signed(31 downto 0) := to_signed(3835,  32);   -- b0 × 2^14
constant A1 : signed(31 downto 0) := to_signed(12549, 32);   -- a1 × 2^14

The timing violation and its fix: The original single-process VHDL chained two 32×32 multiplications, two rounding operations, a 40-bit addition, and a final saturation in a single combinational cloud. The post-implementation timing report showed 22 logic levels, ~13.2 ns worst-case path — failing the 10 ns (100 MHz) constraint by 3.2 ns, with 86 failing endpoints all traced to the de-emphasis source registers.

The fix was to split into two clocked processes with a registered stage boundary:

-- Stage 1: compute raw multiply products, register them
process(aclk)
begin
    if rising_edge(aclk) then
        num_prod_r <= signed(s_axis_data_tdata) * B0;   -- b0 × x[n]
        den_prod_r <= y_prev * A1;                        -- a1 × y[n-1]
        stage1_valid <= s_axis_data_tvalid;
    end if;
end process;

-- Stage 2: round, shift, accumulate, saturate, output
process(aclk)
begin
    if rising_edge(aclk) then
        -- round-shift num_prod_r and den_prod_r to sfix40_En13
        -- add, saturate to sfix32_En13, update y_prev
    end if;
end process;

This halved the combinational depth. Adding one cycle of latency at 50 kHz (2000 clock cycles per sample) is completely imperceptible. The multicycle path constraint (de_emph_mcp.xdc) was applied to the registers spanning the pipeline stages — two iterations were required, as the first constraint missed a second source register (y_prev_reg[*]/C).

The accumulator is sfix40_En13 — wide enough that overflow is analytically impossible for any physically meaningful FM audio signal, and requiring only two DSP48 slices (rather than the three that a sfix48 accumulator would need).

### 5.6 IQ Ping-Pong DMA (iq_ppdma.cpp)

The IQ ppdma core implements free-running DDR→AXI-Stream transfer from alternating ping/pong buffers, with no CPU involvement once started.

Core logic from the actual implementation:

static ap_uint<1>  cur_buf         = 0;   // 0=ping, 1=pong
static ap_uint<32> ridx            = 0;   // read index within buffer
static ap_uint<1>  active_buf_reg  = 0;   // shadow register
#pragma HLS RESET variable=cur_buf
#pragma HLS RESET variable=ridx
#pragma HLS RESET variable=active_buf_reg

// Output stable previous-invocation value FIRST
active_buf = active_buf_reg;

addr_t src_base = (cur_buf == 0) ? ping_base : pong_base;
packed_iq_t packed = mem[(src_base >> 2) + ridx];

// Unpack: I in upper 16 bits, Q in lower 16 bits
i_out = packed(31, 16);
q_out = packed(15,  0);

ridx++;
if (ridx == buf_size_words) {
    ridx    = 0;
    cur_buf = !cur_buf;
}

active_buf_reg = cur_buf;   // commit for next invocation

The shadow register pattern is essential. Without it, HLS schedules the active_buf output as a mux gated to a single FSM state (ap_CS_fsm_state10 in the generated Verilog), driving 'bx on all other states — producing permanent X on the GPIO waveform despite ap_done pulsing correctly. The shadow register (active_buf_reg) stores the committed cur_buf value from the previous invocation, outputting a stable value throughout the current invocation.

The ap_ctrl_hs decision: An earlier version used ap_ctrl_none + PIPELINE II=1. This passed HLS csim but cosim caught a pipelined address-generation hazard: with true II=1 pipelining, iteration N+1 computes its read address while iteration N is still in flight, and the cross-iteration state (cur_buf, ridx) combined with runtime ap_none scalar ports produced an out-of-range address (33, when the maximum valid index was 23). The fix is ap_ctrl_hs — discrete, non-overlapping invocations — with ap_start permanently tied high via xlconstant in the block design.

buf_size_words as a runtime parameter: passed as ap_none (plain port, no AXI-Lite register), driven by a xlconstant in bd.tcl. The value (2500, for 10 ms at 250 kHz) can be changed by updating the xlconstant value and regenerating the bitstream — no HLS resynthesis required.

### 5.7 Audio Ping-Pong DMA (audio_ppdma.cpp)

The audio ppdma core is the mirror image of the IQ ppdma — accepting the free-running 50 kHz audio stream from the de-emphasis filter and writing it to DDR in ping-pong buffers, mirroring each completed 1 ms block to a fixed destination address that software reads.

From the actual implementation’s comment block:

fm_demod’s m_axis_data_0 (de-emphasis output, sfix32_En13, 50 kHz, NO tready / NO tlast) drives this core directly. Because that producer never stalls for backpressure, this core must always finish servicing one incoming sample well inside the ~2000-cycle gap between samples.

The variable-latency nature (1 write per invocation for 49 of every 50 invocations, 50 reads + 50 writes on the 50th) is the reason ap_ctrl_none was rejected. From the actual comment:

Vitis HLS cosim rejects ap_ctrl_none designs whose latency varies per invocation — and this one legitimately does (fast on 49 out of 50 calls, a ~100-beat burst on the 50th).

The m_axi depth parameter has a known cosim pitfall documented in the source:

cosim’s DUMP_INPUTS phase snapshots depth words directly from the pointer BEFORE RTL sim starts, so depth must be <= the actual size of the buffer the testbench allocates — NOT >=. Setting it larger than the real buffer causes cosim to read past the end of the array and segfault during DUMP_INPUTS, before any simulation even runs.

## 6. AXI-Stream Glue Logic

### 6.1 IQ Splitter (iq_splitter.vhd)

Purely combinational. The AXI DMA (and iq_ppdma_0) delivers 32-bit packed I/Q words; freq_corr expects two separate 16-bit streams. The splitter handles the format adaptation and the critical handshake:

m_i_tdata  <= s_tdata(15 downto 0);    -- I in low 16 bits
m_q_tdata  <= s_tdata(31 downto 16);   -- Q in high 16 bits
m_i_tvalid <= s_tvalid;
m_q_tvalid <= s_tvalid;
s_tready   <= m_i_tready AND m_q_tready;  -- both must be ready

The AND-gate on tready ensures neither channel can advance without the other, preventing any I/Q skew that would rotate the discriminator’s phasor.

### 6.2 TLast Generator (tlast_gen.vhd)

The de-emphasis output has no tlast — it is a free-running continuous stream. The AXI DMA S2MM channel requires tlast to know when a transfer is complete; without it, the DMA would wait indefinitely and the CPU would never receive an interrupt.

-- Counter only advances on qualified handshake beats (valid AND ready)
-- Ensures exactly FRAME_SIZE data beats per tlast frame
-- regardless of upstream backpressure
process(aclk)
begin
    if rising_edge(aclk) then
        if aresetn = '0' then
            count <= 1;
        elsif s_tvalid = '1' then
            if count = FRAME_SIZE then count <= 1;
            else                       count <= count + 1;
            end if;
        end if;
    end if;
end process;

m_tlast <= '1' when (s_tvalid = '1' AND count = FRAME_SIZE) else '0';

## 7. Verification: Four-Level Strategy

### 7.1 The Taxonomy of Silent Divergence

The artisan implementation path (translating a Simulink model to HLS/VHDL by hand) introduces a translation step that can produce divergence that is silent (the code compiles and runs), partial (only appears on certain input ranges), and invisible to basic functional tests. There are six distinct classes:

- Wrong fixed-point format — wrong ap_fixed<W,I> types; detected by bit-exact csim comparison

- Wrong rounding/saturation mode — AP_TRN vs AP_RND; introduces DC bias, detectable only by tight-tolerance comparison

- Boundary format error — wrong bit-packing at inter-block interfaces; the sign-extension bug fell into this class

- Control protocol mismatch — ap_ctrl_none vs ap_ctrl_hs; caught by cosim’s latency restrictions

- Reset scope error — static variables not on reset network; X-propagation in simulation only

- Pipeline latency mismatch — wrong cross-correlation offset; produces subtle PSNR degradation

Each level of the test plan is designed to catch a specific class or classes that the other levels cannot.

### 7.2 Level 1 — HLS C Simulation

Each HLS block has a C++ testbench (*_tb.cpp) that: - Loads MATLAB-generated test vectors (from gen_coeffs_testvectors.m) - Calls the HLS function under test with those inputs - Compares output against MATLAB-generated golden values - Prints the first 20 samples and any failures for immediate inspection

The fm_disc testbench uses a critical implementation detail — loading stimulus via .range() rather than floating-point cast:

ic_vals[i].range() = ic_ints[i];   // direct bit assignment, NOT (data_t)double

This assigns the raw integer bit pattern directly, avoiding any rounding in the double→ap_fixed conversion.

### 7.3 Level 2 — HLS Cosim

HLS cosim runs the same C++ testbench against the generated RTL via XSim, with the AXI-Stream/AXI4 protocol automatically handled by generated transactor models. Any mismatch between Level 1 (C sim) and Level 2 (RTL sim) is a synthesis-introduced discrepancy.

Bugs caught at Level 2 in this project: - ap_ctrl_none rejection on audio_ppdma (variable latency) - m_axi depth=1048576 SIGSEGV — cosim reading past the testbench buffer - iq_ppdma pipelined address-generation hazard (out-of-range address 33) - Permanent-X active_buf output on iq_ppdma — traced to missing RESET pragmas

### 7.4 Level 3 — Full Chain Simulation with PSNR Gate

tb_fm_demod_chain.sv connects all six stages as a block design, drives s_axis_i_stimulus.txt / s_axis_q_stimulus.txt (2,500,000 samples of real rds.wav I/Q), captures the audio output, and writes m_axis_data_dut_output.txt.

verify_fm_demod_rtl.m then: 1. Loads the RTL simulation output 2. Trims the startup transient 3. Resamples 50k→48k using resample(audio, 24, 25) 4. Cross-correlates against the Simulink reference (±200 sample search window) to find pipeline latency alignment 5. Computes PSNR against the floating-point reference 6. Applies threshold: >40 dB = PASS

The cross-correlation alignment is essential — the pipeline has a total group delay of several hundred samples that the Simulink model does not account for, so a direct comparison would produce near-zero correlation even when the RTL is correct.

The sign-extension bug was caught at this level: the PSNR was near-zero dB. Adding intermediate waveform probes (aa_lpf_I_y_dut_output.txt, fm_disc_disc_out_dut_output.txt) revealed that the AA LPF was receiving corrupted I/Q values. Tracing backward confirmed that xlconstant_0 was zero-padding the 18-bit sfix18_En17 I and Q values to 24-bit AXI-Stream tdata rather than sign-extending them — meaning all negative values were being converted to large positive numbers before entering the filter. The fix in bd.tcl used xlslice to extract bit 17 (the MSB/sign bit) and xlconcat to replicate it × 6, producing correct two’s-complement sign extension.

### 7.5 Level 4+ — PS7 VIP End-System and Audio Content Check

tb_sdr_fm_receiver_audio_check.sv exercises the complete SoC including both HLS DMA cores, using the PS7 VIP to: 1. Pre-load IQ_PING and IQ_PONG from the same stimulus files via HP0 write_mem 2. Release PL reset 3. Poll audio_ppdma_0’s active_buf via GP0 GPIO reads 4. After each toggle, read AUDIO_DEST_ADDR via HP0 and write to m_axis_data_dut_output.txt

This testbench was developed specifically to reproduce and diagnose the hardware bring-up fault. The simulation runs in a model where HP0 and GP0 share the same DDR with no cache — reads always see what was last written. The result:

250/250 non-zero audio words at AUDIO_DEST_ADDR.

Audio data was present and correct in simulation but wrong on hardware. The only difference between the two environments is the ARM cache. This confirmed the bug was cache coherency — definitively, without ambiguity.

## 8. Hardware Bring-Up: The Cache Coherency Fault

### 8.1 The Symptom

The bare-metal application produced a WAV file of the correct duration (2.5 seconds) but containing no intelligible audio. The correct duration ruled out a control flow failure — the polling loop ran to completion. The wrong content meant the signal processing was incorrect or the data movement was broken.

### 8.2 The Bug

drain_audio_block() in main.c contained:

Xil_DCacheInvalidateRange((INTPTR)AUDIO_DEST_ADDR,
                           AUDIO_SAMPLES_PER_BUFFER * sizeof(u32));
for (u32 i = 0; i < AUDIO_SAMPLES_PER_BUFFER; i++) {
    int32_t raw = (int32_t)Xil_In32(AUDIO_DEST_ADDR + i * sizeof(u32));
    // ... convert to PCM16
}

Xil_In32 is a direct register-style read that bypasses the ARM cache entirely — it reads from the memory controller directly, without consulting or updating the CPU cache. The Xil_DCacheInvalidateRange call therefore had no effect on the reads that followed it, because those reads didn’t use the cache. The cache lines at AUDIO_DEST_ADDR could hold stale power-on content, and neither the invalidate nor the Xil_In32 reads would update them.

audio_ppdma_0 writes via HP0, which also bypasses the ARM cache. In the absence of explicit cache maintenance, there is no mechanism for the CPU’s cached view of AUDIO_DEST_ADDR to be updated with what audio_ppdma_0 wrote.

### 8.3 The Fix

static u32 audio_dest_raw[AUDIO_SAMPLES_PER_BUFFER]
    __attribute__((aligned(64)));

// Invalidate BEFORE reading — marks cache lines as stale
Xil_DCacheInvalidateRange((INTPTR)audio_dest_raw,
                           sizeof(audio_dest_raw));

// Read THROUGH cache via normal pointer — CPU fetches fresh
// lines from DDR into cache after the invalidate
memcpy(audio_dest_raw,
       (const void *)(uintptr_t)AUDIO_DEST_ADDR,
       sizeof(audio_dest_raw));

for (u32 i = 0; i < AUDIO_SAMPLES_PER_BUFFER; i++) {
    int32_t raw = (int32_t)audio_dest_raw[i];
    // ... convert to PCM16
}

After Xil_DCacheInvalidateRange, the CPU’s cache lines covering audio_dest_raw are marked invalid. The next CPU load from that address range triggers a cache refill from DDR — fetching the fresh content that audio_ppdma_0 wrote via HP0. Using memcpy (not Xil_In32) ensures the CPU uses its cached load path, so the invalidate has the intended effect.

refill_iq_buffer() was already correct: memcpy into a DDR staging buffer followed by Xil_DCacheFlushRange(dest_addr, ...) is the right pattern for CPU→PL direction — flush pushes the CPU’s cached writes out to DDR where iq_ppdma_0 can read them via HP0.

### 8.4 How Simulation Isolated the Bug

The PS7 VIP’s HP0 and GP0 ports share the same DDR model in simulation — there is no cache. Reads always see what was last written, regardless of which path wrote it. Running tb_sdr_fm_receiver_audio_check.sv produced 250/250 non-zero audio words at AUDIO_DEST_ADDR. The signal chain was producing correct audio in simulation. The only structural difference between simulation and hardware is the ARM cache — and that difference was the bug.

This demonstrates the key value of the PS7 VIP end-system testbench: it can distinguish signal-processing bugs (wrong audio even in simulation) from implementation bugs (correct in simulation, wrong on hardware). The distinction determines the debugging strategy completely.

## 9. Bare-Metal Application Architecture

### 9.1 The Polling Superloop

The bare-metal application runs without an RTOS. A tight polling loop checks two GPIO bits and does work when they toggle:

// Prime both IQ buffers before entering the superloop
// iq_ppdma_0 starts reading the instant PL reset deasserts (ap_start tied high)
refill_iq_buffer(&fin, IQ_PING_ADDR, &eof_tmp);
refill_iq_buffer(&fin, IQ_PONG_ADDR, &eof_tmp);
iq_prev = 0; audio_prev = 0;

for (;;) {
    u32 iq_now    = Xil_In32(AXI_GPIO_BASE + GPIO_DATA_OFF)  & 0x1;
    u32 audio_now = Xil_In32(AXI_GPIO_BASE + GPIO2_DATA_OFF) & 0x1;

    if (iq_now != iq_prev) {
        u32 free_addr = (iq_now == 0) ? IQ_PONG_ADDR : IQ_PING_ADDR;
        refill_iq_buffer(&fin, free_addr, &eof);
        iq_prev = iq_now;
    }

    if (audio_now != audio_prev) {
        drain_audio_block(&fout, &total_audio_bytes);
        audio_prev = audio_now;
    }

    if (eof && audio_blocks_drained >= AUDIO_EOF_GRACE_BLOCKS) break;
}

The timing contract: iq_ppdma_0 drains one 2500-sample buffer every 10 ms (2500 / 250 kHz). The CPU’s refill_iq_buffer() — an SD card f_read() followed by memcpy and Xil_DCacheFlushRange — must complete within that 10 ms window. audio_ppdma_0 drains one 50-sample buffer every 1 ms (50 / 50 kHz). The CPU’s drain_audio_block() must complete within 1 ms.

### 9.2 The 50k→48k Polyphase Resampler

The FPGA outputs audio at 50 kHz — the only clean integer decimation ratio from 250 kHz that keeps the output above the 48 kHz Nyquist limit. Standard audio equipment expects 44.1 kHz or 48 kHz. The bare-metal software applies a polyphase FIR resampler (L=24, M=25) to convert to 48 kHz before writing the final WAV.

L=24, M=25 → rate conversion ratio = 24/25 = 0.96 (−4%)
Total taps: 2208 (= 24 × 92)
Taps per phase: 92
Cutoff: min(25 kHz, 24 kHz) = 24 kHz
Window: Kaiser β=8, >66 dB stopband attenuation

The resampler matches MATLAB’s resample(audio, 24, 25) output, ensuring the software WAV matches the MATLAB post-processing reference used in verify_fm_demod_rtl.m.

## 10. Linux Port

The PetaLinux application (fmdemod-linux.c) preserves the identical ping-pong DMA logic with the following interface substitutions:

| Bare-metal | Linux |
| --- | --- |
| Xil_DCacheInvalidateRange | O_SYNC mmap — maps DDR as device memory (uncached), no invalidate needed |
| Xil_DCacheFlushRange | Not needed — O_SYNC makes all accesses uncached |
| Xil_In32 for GPIO | gpiod_line_get_value() via libgpiod character device API |
| FATFS f_open/f_read/f_write | POSIX fopen/fread/fwrite |
| Bare superloop | SCHED_FIFO + mlockall() + CPU_SET affinity |
| xparameters.h base addresses | Fixed DDR addresses via mmap /dev/mem |

The O_SYNC pattern for non-coherent DMA:

int devmem_fd = open("/dev/mem", O_RDWR | O_SYNC);   // O_SYNC is load-bearing
volatile uint32_t *audio_dest = mmap(
    NULL, MMAP_SIZE, PROT_READ | PROT_WRITE,
    MAP_SHARED, devmem_fd, AUDIO_DEST_ADDR);

O_SYNC on /dev/mem produces an uncached mapping on ARM Linux — the MMU marks those pages as device memory. Every read goes directly to DDR; every write goes directly to DDR. The CPU never caches the contents of these pages, eliminating the need for explicit cache maintenance.

PetaLinux build challenges documented: - openssh and dropbear conflict — both provide sshd; packagegroup-core-ssh-dropbear is hardcoded in the base image and cannot be removed via rootfs_config alone (requires a bbappend); fix is to use dropbear-scp instead of openssh - resample_50k_to_48k.c not compiled — the scaffolded Makefile only compiles the app’s primary .c file; companion sources must be added to OBJS in a hand-written Makefile and listed in SRC_URI in the .bb recipe - FILE*/ comment bug — FILE* followed by /fopen inside a block comment prematurely terminates the comment; the compiler sees /fopen/fread/fwrite as bare code

## 11. Build Instructions

### Prerequisites

- Vivado / Vitis HLS / PetaLinux 2022.2

- MATLAB R2022b with DSP System Toolbox and Fixed-Point Designer

- Digilent board files for Vivado

### Step 1 — MATLAB: Generate Test Vectors

# In MATLAB:
run_and_extract           % Simulink simulation → ic_qc_gold.mat
gen_coeffs_testvectors    % All HLS testbench vectors + .coe files

### Step 2 — Build HLS IPs

for d in aa_lpf fm_disc audio_ppdma iq_ppdma; do
    cd vivado/$d && bash build.sh && cd ../..
done

Csim and cosim must both pass. The build.sh scripts run vitis_hls -f run_hls.tcl which executes: csim_design → csynth_design → cosim_design → export_design -format ip_catalog.

### Step 3 — Level 3 Simulation (optional, recommended)

cd vivado/fm_demod && bash run_batch_sim.sh
# In MATLAB:
verify_fm_demod_rtl    % must report PSNR > 40 dB

### Step 4 — Build the SoC (headless)

cd vivado/end_system
stdbuf -oL -eL vivado -mode batch -source prj.tcl 2>&1 | tee build.log

Produces sdr_fm_receiver.xsa (hardware export with bitstream).

### Step 5a — Bare-Metal Application

Build in Vitis from the XSA. Add bare_metal/main.c, resample_50k_to_48k.c, resample_coeffs.h. Enable xilffs and xilcache in the BSP. Copy rds.wav to the SD card root. Program the Zybo Z7-10 and monitor UART0 at 115200 8N1.

### Step 5b — PetaLinux

cd petalinux
mkdir -p app
cp fmdemod-linux.c resample_50k_to_48k.c resample_coeffs.h app/
bash build_petalinux.sh         # ~2 hours first build
sudo bash create_sd.sh /dev/sdX path/to/rds.wav

Boot with JP5 in SD mode. Login as root. Run gpiodetect and gpioinfo to verify GPIO chip/line numbers before running fmdemod-linux.

## 12. Resource Utilisation and Timing

### Post-Implementation Results (xc7z010clg400-1)

| Resource | Used | Available | Utilisation |
| --- | --- | --- | --- |
| Slice LUTs | ~4,800 | 17,600 | ~27% |
| Slice Flip-Flops | ~6,200 | 35,200 | ~18% |
| DSP48E1 | 28 | 80 | 35% |
| Block RAM (36 Kb) | 8 | 60 | 13% |
| BUFG | 2 | 32 | 6% |

DSP48 allocation:

| Block | DSP48 | Notes |
| --- | --- | --- |
| AA LPF I-channel | ~9 | Symmetric folding: 65 unique taps / 7 taps per DSP48 |
| AA LPF Q-channel | ~9 | Same filter, second instance |
| FM discriminator cross-products | ~4 | Four 18×18 multiplies |
| FIR decimator | ~3 | Polyphase: 9 taps per phase at R=5 |
| Audio LPF | ~3 | Non-polyphase, full 127 taps at 50 kHz |
| Total | ~28 | 35% of 80 |

This tight fit directly validates the Fixed-Point Designer word-length analysis. Using sfix32_En32 arithmetic throughout (32-bit × 32-bit products) would have required 4 DSP48 slices per multiply rather than 1, consuming approximately 64 DSP48 slices for the four AA LPF and discriminator multipliers alone — before the FIR compiler IPs were added. The design would not fit on the Z7-10.

### Timing

| Metric | Value |
| --- | --- |
| Target clock period | 10.000 ns (100 MHz) |
| WNS after multicycle constraint | +0.432 ns |
| Worst hold slack | +0.071 ns |
| Failing setup paths | 0 |
| Failing hold paths | 0 |
| TNS | 0.000 ns |

The de-emphasis IIR timing closure required two iterations of the multicycle path constraint. The first set_multicycle_path -setup 2 application left 86 failing endpoints because a second source register (y_prev_reg[*]/C) was not included in the constraint’s -from clause. The corrected constraint covering both source registers cleared all failures.

## Appendix: Key Files and Their Roles

| File | Language | Role |
| --- | --- | --- |
| vivado/gen_coeffs_testvectors.m | MATLAB | Master test vector orchestrator (6 steps) |
| vivado/aa_lpf/aa_lpf.h | HLS C++ | Data types: data_t, coef_t, acc_t |
| vivado/aa_lpf/aa_lpf.cpp | HLS C++ | 129-tap FIR, PIPELINE on loop |
| vivado/fm_disc/fm_disc.h | HLS C++ | CORDIC data types: 5 distinct ap_fixed<> |
| vivado/fm_disc/fm_disc.cpp | HLS C++ | CORDIC 16-iter unrolled, 1-sample delay regs |
| vivado/iq_ppdma/iq_ppdma.cpp | HLS C++ | DDR→AXI-S, shadow register, RESET pragmas |
| vivado/audio_ppdma/audio_ppdma.cpp | HLS C++ | AXI-S→DDR, burst mirror, variable latency |
| vivado/freq_corr/freq_corr.vhd | VHDL | Complex multiply, 4-stream all-valid gating |
| vivado/de_emphasis/de_emph.vhd | VHDL | 2-stage IIR pipeline, MCP constraint |
| vivado/de_emphasis/de_emph_mcp.xdc | XDC/Tcl | Multicycle path constraint (2 rounds) |
| vivado/fm_demod_axis_with_sidechannels/iq_splitter.vhd | VHDL | Combinational 32b→two 16b splitter |
| vivado/fm_demod_axis_with_sidechannels/tlast_gen.vhd | VHDL | Periodic tlast insertion for DMA |
| vivado/fm_demod/bd.tcl | Tcl | Inner block design (signal chain) |
| vivado/end_system/bd.tcl | Tcl | Full SoC block design |
| vivado/end_system/prj.tcl | Tcl | Headless build: synth→impl→bitstream→XSA |
| vivado/fm_demod/tb_fm_demod_chain.sv | SystemVerilog | Level 3 chain testbench |
| vivado/fm_demod/verify_fm_demod_rtl.m | MATLAB | PSNR gate: >40 dB = PASS |
| vivado/end_system/tb_sdr_fm_receiver_audio_check.sv | SystemVerilog | PS7 VIP audio content testbench |
| bare_metal/main.c | C | SD card I/O, GPIO polling, cache fix |
| bare_metal/resample_50k_to_48k.c | C | L=24/M=25 polyphase FIR resampler |
| petalinux/build_petalinux.sh | Bash | Automated PetaLinux build |
| petalinux/reserved-memory.dtsi | DTS | no-map DDR reservation |
| petalinux/create_sd.sh | Bash | SD card preparation |
| petalinux/app/fmdemod-linux.c | C | Linux port: O_SYNC mmap, libgpiod |

Marco Aiello · Principal FPGA Design and Verification Engineer
Huntingdon, Cambridgeshire, UK
Vivado / Vitis HLS / PetaLinux 2022.2 · MATLAB R2022b · Zynq-7010